TEAM NO: 01

ROLL NO: SRENIDHI G P 24ECR195

SUNMATHI S 24ECR212

SUBIRAJ B 24ECR202

# Offline Wi-Fi Port Communication between Raspberry Pi and Laptop

PROBLEM STATEMENT:

To create a local communication channel between a Raspberry Pi and a laptop using a WiFi module or hotspot without internet access. The communication will use TCP socket programming and a specific port number so both devices can send and receive messages.

REQUIRED HARDWARE:

- Raspberry Pi (any model with wifi)

- Laptop

- Wi-fi module / mobile hotspot / ESP32 access point

- MicroSD card

REQUIRED SOFTWARE:

- Arduino IDE

- Python3.10(64-bit)

- Raspberry pi OS

- Python Socket Library

- Terminal

WI-FI CONNECTION FOR ESP32:

#include <WiFi.h>

const char* ssid = "Vivo Y28";

const char* password = "_____";

void setup() {

Serial.begin(115200);

Serial.println("Connecting to WiFi...");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

Serial.println("Connecting...");

}

Serial.println("WiFi Connected!");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

}

void loop() {

// Nothing needed here

}

CREATE A SERVER ON RASPBERRY PI:

import socket

host = '10.235.28.211'
port = 5000

server = socket.socket(socket.AF_INET, socket.SOCK_STREAM)
server.bind((host, port))
server.listen(1)

print('Waiting for connection...')

conn, addr = server.accept()
print('Connected by', addr)

while True:
    data = conn.recv(1024)
    if not data:
        break
    print('Received:', data.decode())

Run the server:

python3 server.py

CREATE CLIENT ON LAPTOP:

import socket

host = '10.235.28.211'  # Raspberry Pi IP
port = 5000

client = socket.socket(socket.AF_INET, socket.SOCK_STREAM)
client.connect((host, port))

message = 'Hello from laptop'
client.send(message.encode())
print('Message sent')

## Run the Client

python3 client.py

FOR EITHER SIDE COMMUNICATION

(Raspberry  Laptop):

SERVER CODE:

import socket

host = '10.235.28.211'   # Raspberry pi IP
port = 5000

server = socket.socket(socket.AF_INET, socket.SOCK_STREAM)
server.bind((host, port))
server.listen(1)

print("Waiting for connection...")

conn, addr = server.accept()
print("Connected by", addr)

while True:
    # Receive message
    data = conn.recv(1024)
    if not data:
        break
    print("Laptop:", data.decode())

    # Send reply
    message = input("Pi: ")
    conn.send(message.encode())

conn.close()

CLIENT CODE:

import socket

host = '10.235.28.211'

port = 5000

client = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

client.connect((host, port))

print("Connected to Raspberry Pi")

while True:

# Send message

message = input("Laptop: ")

client.send(message.encode())

# Receive reply

data = client.recv(1024)

print("Pi:", data.decode())

OUTPUT:

OVERALL FLOW:

Setup the devices (lap & rasp) with same wi-fi network

Note the IP address of both devices

For rasp: hostname -I

For lap: run the wi-fi code note the

IP from serial monitor

Make raspberry pi as the Server

Make laptop as the Client

Run the code

Rasp: python3 server.py

Python terminal: python client.py

Communication begins between two devices

without separate network access(thorough

local network)

RESULT:

Thus the ESP32 successfully created a wi-fi Access point through which laptop and raspberry pi connected and communicated each other. Allowing real-time dual side communication between devices without internet access has been done through the specified port (port:5000).